Lab: How to configure SNMPv3 (DAYUTH THY)

323

PC: with PowerSNMP Free Manager

Configuration on Router (Agent side):

!===========================================================================

ip access-list standard PERMIT-SNMP

permit 192.168.1.0 0.0.0.255

Exit

snmp-server engineID local 0000000003

snmp-server group SNMP-G1 v3 priv write SNMP-RW access PERMIT-SNMP

snmp-server view SNMP-RW iso included

snmp-server host 192.168.1.3 version 3 priv SNMP-Admin

snmp-server user SNMP-Admin SNMP-G1 v3 auth sha authpass priv aes 128 pripass

snmp-server enable traps

End

!===========================================================================

R2(config)#snmp-server group SNMP-G1 v3 ?

auth    group using the authNoPriv Security Level

noauth  group using the noAuthNoPriv Security Level

priv    group using SNMPv3 authPriv security level

--------------------------------------

SNMP-Admin: is the username

SNMP-G1: is the group name

authpass: is the password for authentication

pripass: is the password for encryption

!Verify

R2#sh run | s snmp

mmi snmp-timeout 180

snmp-server engineID local 0000000003

snmp-server group SNMP-G1 v3 priv write SNMP-RW access PERMIT-SNMP

snmp-server view SNMP-RW iso included

snmp-server enable traps snmp authentication linkdown linkup coldstart warmstart

snmp-server enable traps vrrp

snmp-server enable traps flowmon

snmp-server enable traps ds1

snmp-server enable traps tty

snmp-server enable traps eigrp

snmp-server enable traps ospf state-change

snmp-server enable traps ospf errors

snmp-server enable traps ospf retransmit

snmp-server enable traps ospf lsa

snmp-server enable traps ospf cisco-specific state-change nssa-trans-change

snmp-server enable traps ospf cisco-specific state-change shamlink interface

snmp-server enable traps ospf cisco-specific state-change shamlink neighbor

snmp-server enable traps ospf cisco-specific errors

snmp-server enable traps ospf cisco-specific retransmit

snmp-server enable traps ospf cisco-specific lsa

snmp-server enable traps xgcp

snmp-server enable traps isdn call-information

snmp-server enable traps isdn layer2

snmp-server enable traps isdn chan-not-avail

snmp-server enable traps isdn ietf

snmp-server enable traps ethernet cfm cc mep-up mep-down cross-connect loop conf                                                                                                                ig

snmp-server enable traps ethernet cfm crosscheck mep-missing mep-unknown service                                                                                                                -up

snmp-server enable traps ethernet evc status create delete

snmp-server enable traps diameter

snmp-server enable traps auth-framework sec-violation auth-fail

snmp-server enable traps trustsec-sxp conn-srcaddr-err msg-parse-err conn-config                                                                                                                -err binding-err conn-up conn-down binding-expn-fail oper-nodeid-change binding-                                                                                                                conflict

snmp-server enable traps aaa_server

snmp-server enable traps atm subif

snmp-server enable traps bfd

snmp-server enable traps bgp cbgp2

snmp-server enable traps bstun

snmp-server enable traps cef resource-failure peer-state-change peer-fib-state-c                                                                                                                hange inconsistency

snmp-server enable traps memory bufferpeak

snmp-server enable traps cnpd

snmp-server enable traps config-copy

snmp-server enable traps config

snmp-server enable traps config-ctid

snmp-server enable traps dial

snmp-server enable traps dlsw

snmp-server enable traps dsp card-status

snmp-server enable traps dsp oper-state

snmp-server enable traps dsp video-usage

snmp-server enable traps dsp video-out-of-resource

snmp-server enable traps entity-ext

snmp-server enable traps entity

snmp-server enable traps fru-ctrl

snmp-server enable traps resource-policy

snmp-server enable traps frame-relay multilink bundle-mismatch

snmp-server enable traps frame-relay

snmp-server enable traps frame-relay subif

snmp-server enable traps hsrp

snmp-server enable traps ipmobile

snmp-server enable traps ipmulticast

snmp-server enable traps isis

snmp-server enable traps mempool

snmp-server enable traps mpls traffic-eng

snmp-server enable traps mpls fast-reroute protected

snmp-server enable traps mpls rfc ldp

snmp-server enable traps mpls ldp

snmp-server enable traps msdp

snmp-server enable traps mvpn

snmp-server enable traps pim neighbor-change rp-mapping-change invalid-pim-messa                                                                                               ge

snmp-server enable traps pppoe

snmp-server enable traps cpu threshold

snmp-server enable traps nhrp nhs

snmp-server enable traps nhrp nhc

snmp-server enable traps nhrp nhp

snmp-server enable traps nhrp quota-exceeded

snmp-server enable traps rsvp

snmp-server enable traps ipsla

snmp-server enable traps stun

snmp-server enable traps l2tun session

snmp-server enable traps l2tun pseudowire status

snmp-server enable traps pw vc

snmp-server enable traps waas

snmp-server enable traps event-manager

snmp-server enable traps firewall serverstatus

snmp-server enable traps ospfv3 state-change

snmp-server enable traps ospfv3 errors

snmp-server enable traps pki

snmp-server enable traps gdoi gm-start-registration

snmp-server enable traps gdoi gm-registration-complete

snmp-server enable traps gdoi gm-re-register

snmp-server enable traps gdoi gm-rekey-rcvd

snmp-server enable traps gdoi gm-rekey-fail

snmp-server enable traps gdoi ks-rekey-pushed

snmp-server enable traps gdoi gm-incomplete-cfg

snmp-server enable traps gdoi ks-no-rsa-keys

snmp-server enable traps gdoi ks-new-registration

snmp-server enable traps gdoi ks-reg-complete

snmp-server enable traps ike policy add

snmp-server enable traps ike policy delete

snmp-server enable traps ike tunnel start

snmp-server enable traps ike tunnel stop

snmp-server enable traps ipsec cryptomap add

snmp-server enable traps ipsec cryptomap delete

snmp-server enable traps ipsec cryptomap attach

snmp-server enable traps ipsec cryptomap detach

snmp-server enable traps ipsec tunnel start

snmp-server enable traps ipsec tunnel stop

snmp-server enable traps ipsec too-many-sas

snmp-server enable traps ethernet cfm alarm

snmp-server enable traps ccme

snmp-server enable traps srst

snmp-server enable traps rf

snmp-server enable traps bulkstat collection transfer

snmp-server enable traps mpls vpn

snmp-server enable traps voice

snmp-server enable traps dnis

snmp-server enable traps vrfmib vrf-up vrf-down vnet-trunk-up vnet-trunk-down

snmp-server host 192.168.111.3 version 3 priv SNMP-Admin

How to configure on PC:

Right-click on SNMP Agents > Add Agent…

Address: 192.168.111.2 is the IP address of SNMP agent installed

Port: 161, Version: 3

Name:  is username

Auth Password: is the password of Authentication, Auth Protol: according to router

Priv Password: is the password of encryption, Priv Protocl: match to router configuration

To configure traps:

Click Tool > Configuration

Click on Edit SNMPv3 Trap Users…

Click Add

Engin ID from command: snmp-server engineID local 0000000003

Name: is the username

Auth Password: is the password of Authentication,

Priv Password: is the password of encryption

Auth Protol: according to router

Priv Protocl: match to router configuration

Address: is IP of PC to listen to trap, Port change to 192 and click OK

To test:

Right click on SysName > Query…

The result of the query is R2, which is the name of the router

Add a loopback interface on R2 by using command below:

R2(config)#interface lo2

R2(config-if)#

*Dec 22 08:30:33.395: %LINEPROTO-5-UPDOWN: Line protocol on Interface Loopback2, changed state to up

Double click on the Trap message under the Traps/Informs Tab of the bottom display pane, it show the Loopback2 is up state

Verify if SNMP traps has been sent from Router:

R2#sh snmp

Chassis: 2048003

14 SNMP packets input

0 Bad SNMP version errors

0 Unknown community name

0 Illegal operation for community name supplied

0 Encoding errors

6 Number of requested variables

0 Number of altered variables

6 Get-request PDUs

0 Get-next PDUs

0 Set-request PDUs

0 Input queue packet drops (Maximum queue size 1000)

20 SNMP packets output

0 Too big errors (Maximum packet size 1500)

0 No such name errors

0 Bad values errors

0 General errors

0 Response PDUs

6 Trap PDUs

SNMP Dispatcher:

queue 0/75 (current/max), 0 dropped

SNMP Engine:

queue 0/1000 (current/max), 0 dropped

0 Unknown Security Models

0 SNMP Invalid Messages

1 SNMP Unknown PDU handlers

0 Unsupported Security Level

4 Unknown User Names

3 Unknown EngineIDs

6 messages have been sent

0 Not In Time Windows

0 Wrong MD5 or SHA Digests

0 Decryption Errors

SNMP logging: enabled

Logging to 192.168.111.3.162, 0/10, 6 sent, 0 dropped.

R2#conf t

Enter configuration commands, one per line.  End with CNTL/Z.

R2(config)#in

R2(config)#interface lo

R2(config)#interface lo2

R2(config-if)#sh

R2(config-if)#no

*Dec 22 08:38:11.443: %LINK-5-CHANGED: Interface Loopback2, changed state to administratively down

*Dec 22 08:38:12.448: %LINEPROTO-5-UPDOWN: Line protocol on Interface Loopback2, changed state to down

R2(config-if)#no sh

R2(config-if)#end

R2#

R2#sh snmp

*Dec 22 08:38:17.117: %SYS-5-CONFIG_I: Configured from console by console

R2#sh snmp

Chassis: 2048003

14 SNMP packets input

0 Bad SNMP version errors

0 Unknown community name

0 Illegal operation for community name supplied

0 Encoding errors

6 Number of requested variables

0 Number of altered variables

6 Get-request PDUs

0 Get-next PDUs

0 Set-request PDUs

0 Input queue packet drops (Maximum queue size 1000)

23 SNMP packets output

0 Too big errors (Maximum packet size 1500)

0 No such name errors

0 Bad values errors

0 General errors

0 Response PDUs

9 Trap PDUs

SNMP Dispatcher:

queue 0/75 (current/max), 0 dropped

SNMP Engine:

queue 0/1000 (current/max), 0 dropped

0 Unknown Security Models

0 SNMP Invalid Messages

1 SNMP Unknown PDU handlers

0 Unsupported Security Level

4 Unknown User Names

10 messages have been sent after sh and no shut has been issued on Loopback 2 interface

3 Unknown EngineIDs

0 Not In Time Windows

0 Wrong MD5 or SHA Digests

0 Decryption Errors

SNMP logging: enabled

Logging to 192.168.111.3.162, 0/10, 10 sent, 0 dropped.

R2#

*Dec 22 08:38:18.067: %LINK-3-UPDOWN: Interface Loopback2, changed state to up

*Dec 22 08:38:19.072: %LINEPROTO-5-UPDOWN: Line protocol on Interface Loopback2, changed state to up

Link to download: https://powersnmp-free-manager.software.informer.com/download/