ASA LAB (THY DAYUTH) Group 1 Group 1

Topology

- Show Run

ciscoasa# show run

: Saved

:

: Serial Number: 9AL5BLSCCN6

: Hardware:   ASAv, 2048 MB RAM, CPU Xeon 5500 series 2809 MHz

:

ASA Version 9.8(3)

!

hostname ciscoasa

enable password $sha512$5000$Vp7lLmpz+JbmGq2jg/z9ZQ==$Zy9UTVep05IBMhYJHamDdQ== pbkdf2

names

no mac-address auto

!

interface GigabitEthernet0/0

nameif outside

security-level 0

ip address 209.165.200.226 255.255.255.248

!

interface GigabitEthernet0/1

nameif dmz

security-level 50

ip address 172.16.33.1 255.255.255.128

!

interface GigabitEthernet0/2

nameif dmz

security-level 50

ip address 172.16.33.1 255.255.255.128

!

interface GigabitEthernet0/2

nameif inside

security-level 100

ip address 172.16.33.129 255.255.255.128

!

interface Management0/0

shutdown

no nameif

no security-level

no ip address

!

ftp mode passive

object network INSIDE_NET

subnet 172.16.33.128 255.255.255.128

object network DMZ_SERVER

host 172.16.33.2

access-list OUTSIDE_IN extended permit tcp any object DMZ_SERVER eq www

access-list OUTSIDE_IN extended permit icmp any any echo-reply

access-list OUTSIDE_IN extended permit icmp any any echo

pager lines 23

mtu outside 1500

mtu dmz 1500

mtu inside 1500

no failover

icmp unreachable rate-limit 1 burst-size 1

no asdm history enable

arp timeout 14400

no arp permit-nonconnected

arp rate-limit 8192

!

object network INSIDE_NET

nat (inside,outside) dynamic interface

object network DMZ_SERVER

nat (dmz,outside) static 209.165.200.227

access-group OUTSIDE_IN in interface outside

route outside 0.0.0.0 0.0.0.0 209.165.200.225 1

timeout xlate 3:00:00

timeout pat-xlate 0:00:30

timeout conn 1:00:00 half-closed 0:10:00 udp 0:02:00 sctp 0:02:00 icmp 0:00:02

timeout sunrpc 0:10:00 h323 0:05:00 h225 1:00:00 mgcp 0:05:00 mgcp-pat 0:05:00

timeout sip 0:30:00 sip_media 0:02:00 sip-invite 0:03:00 sip-disconnect 0:02:00

timeout sip-provisional-media 0:02:00 uauth 0:05:00 absolute

timeout tcp-proxy-reassembly 0:01:00

timeout floating-conn 0:00:00

timeout conn-holddown 0:00:15

timeout igp stale-route 0:01:10

user-identity default-domain LOCAL

aaa authentication login-history

no snmp-server location

no snmp-server contact

crypto ipsec security-association pmtu-aging infinite

crypto ca trustpoint _SmartCallHome_ServerCA

no validation-usage

crl configure

crypto ca trustpool policy

auto-import

telnet timeout 5

ssh stricthostkeycheck

ssh timeout 5

ssh key-exchange group dh-group1-sha1

console timeout 0

dhcpd dns 172.16.33.2

!

dhcpd address 172.16.33.135-172.16.33.150 inside

dhcpd enable inside

!

threat-detection basic-threat

threat-detection statistics access-list

no threat-detection statistics tcp-intercept

dynamic-access-policy-record DfltAccessPolicy

!

class-map inspection_default

match default-inspection-traffic

!

!

policy-map type inspect dns preset_dns_map

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

policy-map global_policy

class inspection_default

inspect ip-options

inspect netbios

inspect rtsp

inspect sunrpc

inspect tftp

inspect xdmcp

inspect dns preset_dns_map

inspect ftp

inspect h323 h225

inspect h323 ras

inspect rsh

inspect esmtp

inspect sqlnet

inspect sip

inspect skinny

inspect icmp

policy-map type inspect dns migrated_dns_map_2

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

policy-map type inspect dns migrated_dns_map_1

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

!

service-policy global_policy global

prompt hostname context

call-home reporting anonymous prompt 2

call-home

profile CiscoTAC-1

no active

destination address http https://tools.cisco.com/its/service/oddce/services/DDCEService

destination address email callhome@cisco.com

destination transport-method http

subscribe-to-alert-group diagnostic

subscribe-to-alert-group environment

subscribe-to-alert-group inventory periodic monthly

subscribe-to-alert-group configuration periodic monthly

subscribe-to-alert-group telemetry periodic daily

Cryptochecksum:388a298f3d330a436adb11b92f8511a2

: end

- Show Run SSH

- Show route

- Show nat

- Show xlate

- Show access-list

- Show run access-group